START

Hobbs & tach	BOOK Passenger briefing

Seat belts

Shoulder harness

Doors

Windows

No smoking

Egress

+Xcg of flight controls

Destination

Fuel selector	BOTH

Trim	SET

Mixture	IN

Throttle	SET

Carb heat	IN

Circuit breakers	IN

All lights	OFF

Nav/ADSB	OFF

Beacon	ON

Field	OFF

Avionics master	OFF

Master switch	ON

Primer                  	AS REQ

Propeller area	CLEAR

Start (push key in)

Engine gauges	CHECK

Field	ON

Check volts	14+

Avionics master	ON

Nav/ADSB	ON

Set Xpndr                ALT / 1200

Avionics	SET

Altimeter	±75

Start checklist       COMPLETE

N8338X Rev 12/21/2023

RUNUP

Mag check	1600,

100 drop,

50 match

Suction	4+

Carb heat	Idle

Flight instruments	SET

Controls & trim	CHECK

GPS	Destination

Nav/ADSB 	ON

Transponder	ON

Lights	ON

Emergency procedures

47 kts at ½ takeoff distance

land remaining runway

straight ahead ±30°

hard turn, flaps as needed

TAKEOFF

Normal

Climb 	68-78

Short Field

Flaps 	10°

Rotate	50 / 52 / 68

CRUISE

Landing light	OFF

Power	SET

Trim	ADJUST

Mixture	LEAN

Flight instruments	CHECK

###### Engine instruments	CHECK

Transponder

Emergency 7700

NORDO 7600

Hijack 7500

VFR (normal) 1200

PREFLIGHT

Paperwork

Control lock		REMOVE

Fuel selector		BOTH

Left fuel sump

Right fuel sump

Gascolator sump

Oil			6+

Right fuel tank	DIP

Left fuel tank		DIP

Baggage door 	LOCKED

Tail tiedown

Control surfaces

Right aileron

Right wingtip

Right tiedown

Right main wheel

Propeller & spinner

Landing lights

Air filter

Nose gear		BOUNCE

Static port

Pitot

Fuel vent

Stall warning port

Left tiedown

Left wingtip

Left aileron

Left main wheel

Night / IFR

Master switch		ON

IFR Pitot heat		WARM

Taxi light

Landing light

Strobes & nav lights

Instrument lights

Master switch		OFF

X-wind		12 kts

Best Glide		68 kts

VS0		51 kts

VFE		87 kts

VNO		122 kts

VNE		139 kts

VA	            100 @ 2250#

VX		52 kts

VY		68 kts

Fuel		39 gal usable

Tires		35 / 23 psi

N8338X Rev 12/21/2023

Takeoff Performance

| Increase distance 10% for each 15° C above STP for particular altitude. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Increase distance 10% for each 25° F above STP for particular altitude. |  |  |  |  |  |  |  |
| Flaps Up, |  | Sea Levl, 15° C |  | 2500 ft, 10° C |  | 5000 ft, 5° C |  |
| Hard Surface |  | Sea Levl, 59° F |  | 2500 ft, 50° F |  | 5000 ft, 41° F |  |
| Gross Wt | Head Wind | Ground Run | 50' Obst. | Ground Run | 50' Obst. | Ground Run | 50' Obst. |
|  | 0 | 560 | 1000 | 675 | 1185 | 820 | 1420 |
| 1900 | 13 | 335 | 675 | 415 | 805 | 515 | 980 |
|  | 26 | 165 | 400 | 210 | 490 | 275 | 610 |
|  | 0 | 825 | 1430 | 995 | 1710 | 1220 | 2100 |
| 2250 | 13 | 520 | 995 | 640 | 1205 | 795 | 1500 |
|  | 26 | 280 | 625 | 355 | 770 | 455 | 985 |

Cruise and Range Data

| Standard Atmospheric Conditions, no wind. 39 gal. fuel, no reserve, gross wt. 2250 lbs. Rich Mixture |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Altitude | RPM | %BHP | TAS | gal. / hr. | End. hr. | Range nm |
| 2500 | 2700 | 91 | 118 | 12.6 | 3.1 | 365 |
|  | 2600 | 81 | 112 | 11.1 | 3.5 | 391 |
|  | 2500 | 72 | 107 | 10.1 | 3.9 | 413 |
|  | 2400 | 64 | 102 | 9.2 | 4.2 | 430 |
|  | 2300 | 57 | 96 | 8.5 | 4.6 | 439 |
|  | 2200 | 50 | 90 | 7.8 | 5.0 | 452 |
|  | 2100 | 45 | 85 | 7.2 | 5.4 | 460 |
| 5000 | 2700 | 84 | 116 | 21 | 3.2 | 378 |
|  | 2600 | 76 | 112 | 10.9 | 3.6 | 400 |
|  | 2550 | 71 | 109 | 10.4 | 3.8 | 408 |
|  | 2500 | 67 | 106 | 9.9 | 3.9 | 417 |
|  | 2400 | 60 | 99 | 9.1 | 4.3 | 425 |
|  | 2300 | 54 | 95 | 8.4 | 4.6 | 439 |
|  | 2200 | 47 | 90 | 7.7 | 5.1 | 452 |
|  | 2100 | 42 | 84 | 7.1 | 5.5 | 465 |

APPROACH

Wx	     COPY

Altimeter	     SET

DG	     CHECK

Landing light	     ON

Mixture	     IN

Carb heat	     OUT

LANDING

No Flaps	     60-70

Normal	     56-65

Short field     40°     52

CLEANUP

Flaps	UP

Carb heat	IN

Xpndr                      1200

Lights                       AS REQ

SHUTDOWN

All lights	OFF

Nav/ADSB 	   OFF

Avionics master	OFF

Field	OFF

Mixture             	OUT

Master switch	OFF

Ignition switch	OFF

Hobbs & tach	BOOK

Control lock	IN

Flight plan	CLOSE

| Signal | Ground | Air |
| --- | --- | --- |
| green | takeoff | land |
| flash grn | taxi | return |
| red | stop | give way |
| flash red | exit rwy | unsafe |
| Flashwht | return |  |
| red / grn | caution |  |

N8338X Rev 12/21/2023

ENGINE LOSS

Airspeed	68

Fuel selector	BOTH

Mixture	IN

Carb heat	OUT

Ignition switch	BOTH

Primer	IN & LOCKED

LANDING W/O POWER

Airspeed	68

Fuel selector	OFF

Mixture	OUT

Ignition switch	OFF

Flaps	AS REQ

Master switch	OFF

Door	UNLATCH

ENGINE FIRE

Fuel selector	OFF

Mixture	OUT

Throttle	OUT

Airspeed             120 / AS REQ

Cabin heat	CLOSED

Outside vents	OPEN

ELECTRICAL FIRE

Master switch 	OFF

Ignition switch	ON

Other switches	OFF

Cabin heat 	CLOSED

Outside vents 	OPEN

Fire extinguisher 	USE

CABIN FIRE

Master switch 	OFF

Cabin heat 	CLOSED

Outside vents 	CLOSED