| Takeoff Data |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Increase distance 10% for each 15° C above Standard Temperature for particular altitude. |  |  |  |  |  |  |  |  |  |
|  | Increase distance 10% for each 25° F above Standard Temperature for particular altitude. |  |  |  |  |  |  |  |  |  |
| Flaps Up, Hard Surface |  |  | At Sea Level & 15° C |  | At 2500 ft. & 10° C |  | At 5000 ft. & 5° C |  | At 7500 ft. & 0° C |  |
|  |  |  | At Sea Level & 59° F |  | At 2500 ft. & 50° F |  | At 5000 ft. & 41° F |  | At 7500 ft. & 32° F |  |
| Gross Wt | IAS at 50 ft. | Head Wind | Ground Run | to Clear 50' Obst. | Ground Run | to Clear 50' Obst. | Ground Run | to Clear 50' Obst. | Ground Run | to Clear 50' Obst. |
|  |  | 0 | 380 | 725 | 460 | 845 | 555 | 1000 | 680 | 1205 |
| 1600 | 49 | 13 | 215 | 470 | 265 | 560 | 330 | 670 | 415 | 820 |
|  |  | 26 | 95 | 265 | 125 | 320 | 160 | 395 | 210 | 495 |
|  |  | 0 | 560 | 1000 | 675 | 1185 | 820 | 1420 | 1015 | 1755 |
| 1900 | 53 | 13 | 335 | 675 | 415 | 805 | 515 | 980 | 645 | 1230 |
|  |  | 26 | 165 | 400 | 210 | 490 | 275 | 610 | 360 | 785 |
|  |  | 0 | 825 | 1430 | 995 | 1710 | 1220 | 2100 | 1520 | 2650 |
| 2250 | 58 | 13 | 520 | 995 | 640 | 1205 | 795 | 1500 | 1010 | 1920 |
|  |  | 26 | 280 | 625 | 355 | 770 | 455 | 985 | 600 | 1290 |

| Climb Data |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Flaps up, full throttle, mixture leaned for smooth operation above 5000 ft. |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel used includes warm-up and takeoff allowance. |  |  |  |  |  |  |  |  |  |  |  |
| Gross Wt. | At Sea Level & 15° C |  |  | At 5000 ft. & 5° C |  |  | At 10000 ft. & -5° C |  |  | At 15000 ft. & 10° C |  |  |
|  | At Sea Level & 59° F |  |  | At 5000 ft. & 41° F |  |  | At 10000 ft. & 23° F |  |  | At 15000 ft. & 50° F |  |  |
|  | Best Climb IAS | Rate of Climb ft/min | Gal. used | Best Climb IAS | Rate of Climb ft/min | Gal. used | Best Climb IAS | Rate of Climb ft/min | Gal. used | Best Climb IAS | Rate of Climb ft/min | Gal. used |
| 1600 | 62 | 1220 | 1 | 60 | 955 | 1.8 | 58 | 690 | 2.6 | 56 | 425 | 3.8 |
| 1900 | 65 | 940 | 1 | 63 | 710 | 2.1 | 62 | 475 | 3.3 | 60 | 245 | 5.2 |
| 2250 | 68 | 700 | 1 | 67 | 490 | 2.5 | 66 | 285 | 4.4 | 65 | 80 | 8.5 |

| Landing Data |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Approach IAS 56kts @ 2250 lbs. Approach IAS 46kts @ 1600 lbs. Flaps 40°, power off, hard surface runway. Reduce landing distance 10% for each 5kts headwind. All distances landing over a 50' obstacle. |  |  |  |  |  |  |  |
| Gross Wt. | At Sea Level & 15° C |  | At 2500 ft. & 10° C |  | At 5000 ft. & 5° C |  | At 7500 ft. & 0° C |  |
|  | At Sea Level & 59° F |  | At 2500 ft. & 50° F |  | At 5000 ft. & 41° F |  | At 7500 ft. & 32° F |  |
|  | Landing Dist. | Ground Roll | Landing Dist. | Ground Roll | Landing Dist. | Ground Roll | Landing Dist. | Ground Roll |
| 1600 | 870 | 530 | 915 | 560 | 960 | 585 | 1010 | 615 |
| 2250 | 1140 | 690 | 1205 | 730 | 1270 | 775 | 1340 | 815 |

| Cruise and Range Data |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Standard Atmospheric Conditions, no wind. 39 gal. fuel, no reserve, gross wt. 2250 lbs. |  |  |  |  |  |
| Rich Mixture |  |  |  |  |  |  |
| Alt. | RPM | %BHP | TAS | gal. / hr. | End. hr. | Range nm |
| 2500 | 2700 | 91 | 118 | 12.6 | 3.1 | 365 |
|  | 2600 | 81 | 112 | 11.1 | 3.5 | 391 |
|  | 2500 | 72 | 107 | 10.1 | 3.9 | 413 |
|  | 2400 | 64 | 102 | 9.2 | 4.2 | 430 |
|  | 2300 | 57 | 96 | 8.5 | 4.6 | 439 |
|  | 2200 | 50 | 90 | 7.8 | 5.0 | 452 |
|  | 2100 | 45 | 85 | 7.2 | 5.4 | 460 |
| 5000 | 2700 | 84 | 116 | 21 | 3.2 | 378 |
|  | 2600 | 76 | 112 | 10.9 | 3.6 | 400 |
|  | 2550 | 71 | 109 | 10.4 | 3.8 | 408 |
|  | 2500 | 67 | 106 | 9.9 | 3.9 | 417 |
|  | 2400 | 60 | 99 | 9.1 | 4.3 | 425 |
|  | 2300 | 54 | 95 | 8.4 | 4.6 | 439 |
|  | 2200 | 47 | 90 | 7.7 | 5.1 | 452 |
|  | 2100 | 42 | 84 | 7.1 | 5.5 | 465 |

| Airspeed Correction Table |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flaps Up |  |  |  |  |  |  |  |  |  |
| IAS | 35 | 43 | 52 | 61 | 70 | 78 | 87 | 96 | 104 |
| CAS | 51 | 54 | 57 | 63 | 70 | 78 | 87 | 94 | 103 |
| Flaps Down |  |  |  |  |  |  |  |  |  |
| IAS | 35 | 43 | 52 | 61 | 70 | 78 | 87 |  |  |
| CAS | 43 | 50 | 56 | 63 | 71 | 79 | 87 |  |  |

| Stall Speeds |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Power off, gross weight, CAS. |  |  |  |
| Condition | Bank Angle |  |  |  |
|  | 0° | 20° | 30° | 40° |
| Flaps up | 50 | 52 | 58 | 72 |
| Flaps 10° | 47 | 49 | 54 | 67 |
| Flaps 40° | 45 | 47 | 51 | 63 |

| Cruise and Range Data |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Standard Atmospheric Conditions, no wind. 39 gal. fuel, no reserve, gross wt. 2250 lbs. |  |  |  |  |  |  |
| Lean Mixture |  |  |  |  |  |  |  |
| Alt. | RPM | %BHP | TAS | gal. / hr. |  | End. hr. | Range nm |
| 2500 | 2700 | 91 | 119 |  | 11.5 | 3.4 | 404 |
|  | 2600 | 81 | 114 |  | 10.3 | 3.8 | 434 |
|  | 2500 | 72 | 109 |  | 9.2 | 4.2 | 461 |
|  | 2400 | 64 | 103 |  | 8.1 | 2.8 | 495 |
|  | 2300 | 57 | 97 |  | 7.2 | 5.4 | 526 |
|  | 2200 | 50 | 92 |  | 6.4 | 6.1 | 560 |
|  | 2100 | 45 | 87 |  | 5.7 | 6.8 | 595 |
| 5000 | 2700 | 84 | 118 |  | 10.7 | 3.7 | 434 |
|  | 2600 | 76 | 114 |  | 9.6 | 4.1 | 460 |
|  | 2550 | 71 | 110 |  | 9.0 | 4.3 | 478 |
|  | 2500 | 67 | 108 |  | 8.5 | 4.6 | 491 |
|  | 2400 | 60 | 102 |  | 7.6 | 5.1 | 521 |
|  | 2300 | 54 | 96 |  | 6.8 | 5.7 | 556 |
|  | 2200 | 47 | 91 |  | 6.0 | 6.5 | 591 |
|  | 2100 | 42 | 86 |  | 5.4 | 7.2 | 621 |
| 7500 | 2650 | 74 | 115 |  | 9.4 | 4.2 | 478 |
|  | 2600 | 70 | 112 |  | 8.9 | 4.4 | 491 |
|  | 2500 | 63 | 107 |  | 7.9 | 4.9 | 521 |
|  | 2400 | 56 | 101 |  | 7.1 | 5.5 | 552 |
|  | 2300 | 50 | 96 |  | 6.4 | 6.1 | 587 |
|  | 2200 | 45 | 90 |  | 5.7 | 6.8 | 617 |
|  | 2100 | 41 | 86 |  | 5.2 | 7.8 | 652 |
| 10,000 | 2650 | 69 | 114 |  | 8.7 | 4.5 | 508 |
|  | 2600 | 65 | 110 |  | 8.3 | 4.7 | 521 |
|  | 2500 | 59 | 105 |  | 7.4 | 5.2 | 552 |
|  | 2400 | 53 | 100 |  | 6.7 | 5.8 | 582 |
|  | 2300 | 47 | 95 |  | 6.0 | 6.5 | 617 |
|  | 2200 | 43 | 90 |  | 5.4 | 7.2 | 647 |
|  | 2100 | 39 | 87 |  | 5.0 | 7.8 | 678 |
| 12,500 | 2600 | 61 | 109 |  | 7.7 | 5.0 | 552 |
|  | 2500 | 55 | 104 |  | 7.0 | 5.6 | 582 |
|  | 2400 | 50 | 99 |  | 6.3 | 6.2 | 608 |
|  | 2300 | 45 | 95 |  | 5.7 | 6.8 | 643 |
|  | 2200 | 41 | 90 |  | 5.2 | 7.4 | 669 |